<Device API>

- S7, X70EX, N5 Premium

- Inpout32.dll

- _stdcall  function

bool Set_WifiPower(short iPower);

- Wifi On/Off

- On : 1, Off : 0

bool Set_BluePower(short iPower);

- Bluetooth On/Off

- On : 1, Off : 0

bool Set_CameraPower(short iPower);

- Camera On/Off

- On : 1, Off : 0

bool Set_ModemPower(short iPower);

- Modem On/Off

- On : 1, Off : 0

bool Set_DmbPower(short iPower);

- DMB On/Off

- On : 1, Off : 0

char Get_DevicePower();

- Device Power Status (Wifi, Bluetooth, Camera, Modem, DMB)

- Display the binary value.

bool Set_BacklightLevel_ACK(short level);

bool Set_BacklightLevel(short level);

- Change the backlight brightness (0 – 15)

- Call Set_BacklightLevel_ACK when executing first.

- Call Set_BacklightLevel when executing second.

bool Set_PivotInfo(short rotation);

- Send the pivot status to the EC.

- 0 : 0, 1 : 90, 2 : 180, 3 : 270

bool Set_SoundOut(short iPower);

- 1 : Speaker Out, 0 : Line Out

- S7, X70EX

bool Set_KeyRemap(short iPower);

- 0 : <Fn + Ctrl>, 1 : <Ctrl + Fn>

- S7, X70EX

bool Haptic_Control(BYTE onoff, BYTE duration, BYTE mode);

- Set the option of the vibration.

- X70EX

|  | onoff | duration | mode |
| --- | --- | --- | --- |
| Not Use | 0x55 | 0x01 ~ 0xFF | 0x50 |
| Only SoftKeyboard | 0xAA | 0x01 ~ 0xFF | 0x51 |
| Always | 0xAA | 0x01 ~ 0xFF | 0x52 |

bool ExecuteSWHaptic();

- Execute the vibration.

- X70EX